Stage 1   -   Semester 2

May 2023

COMP18025 Networking

Internal Examiner(s):	Paula Duffy

External Examiner(s):	xxxxxxx

Date:

Time:

Duration:

Venue:

No. Pages:			4

Instructions

Answer Any 4 Questions.  Each question is worth 25 marks.Total marks on paper is 100.

PLEASE NOTE:  Exam Paper printed DOUBLE SIDED.

DO NOT TURN THIS PAGE UNTIL INSTRUCTED

Question 1.

- Differentiate between Bandwidth and Throughput in networks.

(4 marks)

- In terms of networking media, what is noise?

(2 marks)

- Compare and Contrast Fibre Optic and Twisted Pair cabling. Your answer should include the characteristics of each, advantages and disadvantages.  Discuss their approaches to noise prevention. Include relevant diagrams.

- marks)

- Discuss Structured Cabling as implemented in DKIT.

(5 marks)

Question  2.

- What is the purpose of the OSI model? In your answer briefly explain its layers and structure.

- marks)

- Outline the layers of the TCP/IP reference model.  Explain what encapsulation refers to and identify the PDUs [Protocol Data Units] used at each layer.

- marks)

(c) A DNS request has been issued from a PC over an ethernet network using UDP. Draw the encapsulated packet.

(4 marks)

Question 3

  - What is a MAC address? Explain how MAC addresses are used in Ethernet networks.

(2 marks)

  - Draw the Ethernet II frame.  Clearly show the size of each field within the frame.

(7 marks)

Figure 1 Wireshark Dump

  - Examine the hex data shown in Figure 1.  This is a HTTP request for the web page www.rte.ie. Decode the Ethernet frame and answer the following:-

    - Which address is the frame being sent to? Is this address a Unicast, Broadcast or Multicast?

(2 marks)

    - Which address sent it? Is this address a Unicast, Broadcast or Multicast?

(2 marks)

    - What is the next upper layer protocol? Describe in detail the function of this protocol.

(6 marks)

  - If the polynomial generator is   , calculate the CRC for the data element 11100101.

(6 marks)

Question 4.

- Two devices used to extend a LAN are Hubs and Switches.  Describe the features of both explaining how they operate and their associated OSI layer.

(6 marks)

- Examine the network in Figure 1.  What kind of topology is being used in the network?

(1 marks)

- Consider the following campus network connecting three access layer (layer 2) switches (Switch X, Switch Y and Switch Z) connecting host clients (A, B, C, D, E and F) with a Web Server (WS) and File Server (FS) in Figure 1. Assume no VLANS have been configured on this network.

Figure 1 Network Diagram

Question 4c continued on next page

Question 4c continued

Using Table 1 below as a guide, update the Mac-Address tables on each of the switches after the following communications. Assume ARP caches are populated, and MAC Address tables are empty in this instance:

- After A sends a web request to the Web Server (WS).

- After the Web Server (WS) replies to A.

- After F sends a web request to the Web Server (WS).

- After the Web Server (WS) replies to F.

- After C sends a web request to the File Server (FS).

- After the File Server (FS) replies to C.

- After E sends a web request to the File Server (FS).

- After the File Server (FS) replies to E.

| Scenario | Switch X |  | Switch Y |  | Switch Z |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MAC (device) | Port | MAC (device) | Port | MAC (device) | Port |
| - 3 |  |  |  |  |  |  |
| - 4 |  |  |  |  |  |  |
| - 6 |  |  |  |  |  |  |
| - 7 |  |  |  |  |  |  |
| - 8 |  |  |  |  |  |  |

Table 1 Mac-Address Tables for each switch

- marks)

- What is a VLAN? Discuss 3 reasons to include VLANs in your network design.

(10 marks)

Question 5.

- Indicate the role of a carrier signal in analogue transmission.

(4 marks)

- Indicate which characteristics of an analog signal are changed to represent the digital signal in each of the following digital-to-analog conversion?

  - ASK

  - FSK

  - PSK

  - QAM

(4 marks)

(c) Explain why synchronisation is important in data communications.

(5 marks)

(d) Draw a graph of the data stream 1110101, assuming that the last signal level has been positive, for the following encoding schemes: -

  - NRZ – I

  - Manchester Encoding

(12 marks)