# DUNDALK INSTITUTE OF TECHNOLOGY

BSc (Hons) in Computing Systems and Operations - Semester 2

# Networking

# Practical Exam 2025

Frances Byrne

Date

Time

Answer ALL questions. Instructions Answers (text and screenshots) to specific questions should be added to a word document and uploaded to Moodle along with the Packet Tracer file..

Figure 1 Practical Exam Topology

- Build the topology shown in Figure 1 using Packet Tracer.

Note

- The switches should be 2960 series switches.

- Connect the PC’s to the switch ports identified in Table 1 and Table 2.

- Connect the switches using port GigabitEthernet0/1 on both switches.

(10 marks)

- Assign the IP configuration shown in Table 1 and Table 2 to each PC.

(12 marks)

| Devices connected to SW-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Switch port | VLAN | IP Address | Subnet Mask |
| Admin PC1 | FastEthernet0/1 | 200 | 192.168.200.1 | 255.255.255.0 |
| Admin PC2 | FastEthernet0/2 |  | 192.168.200.2 | 255.255.255.0 |
| Student PC1 | FastEthernet0/3 | 201 | 192.168.201.1 | 255.255.255.0 |
| Student PC2 | FastEthernet0/4 |  | 192.168.201.2 | 255.255.255.0 |
| Staff PC1 | FastEthernet0/5 | 202 | 192.168.202.1 | 255.255.255.0 |
| Staff PC2 | FastEthernet0/6 |  | 192.168.202.2 | 255.255.255.0 |

Table 1 Configuration information for PC's connected to SW-1

| Devices connected to SW-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Switch port | VLAN | IP Address | Subnet Mask |
| Admin PC3 | FastEthernet0/1 | 200 | 192.168.200.3 | 255.255.255.0 |
| Admin PC4 | FastEthernet0/2 |  | 192.168.200.4 | 255.255.255.0 |
| Student PC3 | FastEthernet0/3 | 201 | 192.168.201.3 | 255.255.255.0 |
| Student PC4 | FastEthernet0/4 |  | 192.168.201.4 | 255.255.255.0 |
| Staff PC3 | FastEthernet0/5 | 202 | 192.168.202.3 | 255.255.255.0 |
| Staff PC4 | FastEthernet0/6 |  | 192.168.202.4 | 255.255.255.0 |

Table 2 Configuration information for PC's connected to SW-2

- Configure the following on the switch SW-1

    - Hostname (SW-1)

(2 marks)

    - Privileged Secret (class)

(2 marks)

    - Console password (cisco)

(2 marks)

    - VTY password (letmein)

(2 marks)

    - Encrypt all passwords.

(2 marks)

    - Create a MOTD banner stating that unauthorised access is prohibited.

(2 marks)

    - Assign the IP address 192.168.1.254 and subnet mask 255.255.255.0 to SW-1 using VLAN 1.

(4 marks)

- Add a screenshot to this document showing where the Console password is required.

(4 marks)

- Configure the following on the switch SW-2

    - Hostname (SW-2)

(2 marks)

    - Assign the IP address 192.168.1.253 and subnet mask 255.255.255.0 to SW-2 using VLAN 1.

(4 marks)

- Add a screenshot to this document showing a ping from SW-1 to SW-2.

(4 marks)

- Configure the 3 VLANs shown in Table 3 on BOTH SW-1 and SW-2

(12 marks)

| VLAN ID | VLAN Name |
| --- | --- |
| 200 | Admin |
| 201 | Student |
| 202 | Staff |

Table 3 VLAN details

- Assign ports to the appropriate VLAN as specified in Table 1 and Table 2.

(12 marks)

- Configure GigabitEthernet0/1 on both SW-1 and SW-2 to permit all VLAN traffic.

(6 marks)

- Using simple PDU’s, determine which of the following tests are successful.

| Test | Successful | Failed |
| --- | --- | --- |
| Staff PC1 Staff PC4 | ✅ |  |
| Staff PC2 Student PC2 |  | ✅ |
| Staff PC3 SW-2 |  | ✅ |
| Admin PC1 Admin PC2 | ✅ |  |
| Admin PC1 Admin PC3 | ✅ |  |
| Admin PC1 Staff PC2 |  | ✅ |

(6 marks)

- Take a screenshot of the ARP table on Admin PC1 and add it to this document.

(4 marks)

- Indicate why some of the tests in part 10 were not successful.

(6 marks)

- Save the configuration on both SW-1 and SW-2.

(2 marks)

Save this file and the Packet Tracer file.  Upload both files to Moodle using the Upload Completed Practical Exam assignment upload facility.